SYMETRIX WORK INSTRUCTIONS

75-0004 – Asm, Repair Kit, xIn 12 / xOut 12 Power Supply

B

A

INCLUDED PARTS

D

C

1x 11-0344-1 Adapter Plate

- 4x 11-0345-1 Screw, Phillips, 
Flat Head

- 1x 11-1661-B Power 
Supply Insulator

- 12-0063 Power Supply, 30W, 24V

E

F

40-0048-B AC Input Cable Assembly (P/N for complete IEC assembly, only the connector and brown and blue wires are needed.)

- 40-0052-A DC Output 
Cable Assembly

## REQUIRED TOOLS

## Philips screwdriver

## Needle nose pliers (optional)

## IMPORTANT

This procedure should only be performed 
by a qualified technician.

## PROCEDURE

STEP 1. Disconnect AC power cable from unit.

STEP 2. Remove top cover, 3 screws at rear.

STEP 3. Disconnect brown and blue wires from IEC 
AC power jack. This can be done by hand, optionally 
use needle nose pliers. Do not disconnect Ground wire.

STEP 4. Disconnect DC power connector 
from main PCB.

STEP 8. Install Power Supply (12-0063) with the 4 Phillips screws removed at Step 5. Note the orientation, 
3 pin AC input toward the IEC AC power connector.

STEP 5. Remove 
the 4 screws securing the power supply and set aside. Remove power supply and insulating material.

STEP 9. Install 
DC Output Cable Assembly 
(40-0052-A).

STEP 6. Install Adapter Plate (11-0344-1) with the 4 Phillips Flat Head Screws (11-0345-1).

STEP 10. Install AC Input Cable Assembly (40-0048-B) ensuring the brown wire is connected to the terminal nearer the unit’s interior, and blue to the terminal nearer the unit’s sidewall.

STEP 7. Install 
Power Supply Insulator (11-1661-B).

STEP 11. Reattach the top cover.